# SDR Hunter — Hardware Bring-Up Guide

This guide takes you from a freshly-plugged-in SDR to a validated, running SDR Hunter install. It is meant to be run on the machine the SDR is physically attached to — not in a headless/mock environment. Every step is ordered so that if something fails, you stop at the lowest layer that’s broken instead of chasing a symptom three layers up.

Legend: $ = run as your normal user, # = needs sudo/root. Supported hardware: BladeRF, RTL-SDR / NooElec, HackRF, LimeSDR, PlutoSDR, USRP (UHD), SDRplay, Airspy — all via SoapySDR.

## Layer 0 — Prerequisites & driver install

- Install SoapySDR + per-device drivers + udev rules using the bundled installer (Debian/Ubuntu, Fedora, Arch, macOS all handled):

- $ cd /path/to/sdr_hunter
$ ./install_drivers.sh

- This installs soapysdr-tools, the Python bindings (python3-soapysdr), and the driver module for each supported radio. SoapySDR Python bindings come from the system package, NOT pip — do not pip install them.

- Install the app’s Python deps:

- $ pip install -r requirements.txt
# optional decoders/extras (weather-sat, drone-id, etc.):
$ pip install -r requirements_optional.txt

- Re-plug the device after install so the new udev rules apply, OR reload them without unplugging:

- # udevadm control --reload-rules && udevadm trigger

## Layer 1 — Is the OS seeing the USB device?

Before any SDR software, confirm the kernel enumerated the hardware.

$ lsusb

Look for your radio in the list, e.g.:

| Device | Typical lsusb string |
| --- | --- |
| RTL-SDR | Realtek ... RTL2838 DVB-T |
| HackRF | Great Scott Gadgets HackRF One |
| BladeRF | Nuand ... bladeRF |
| LimeSDR | Lime Microsystems LimeSDR-USB |
| Airspy | Airspy |
| PlutoSDR | Analog Devices Inc. PlutoSDR (ADALM-PLUTO) |

If it does not appear: it’s a cable/power/USB-port problem, not a software one. Try a different (data-capable) cable, a rear/powered USB port, and avoid unpowered hubs. BladeRF/LimeSDR/USRP draw significant current — use USB 3.0.

RTL-SDR only: blacklist the kernel DVB-T driver or it will grab the device:

# echo 'blacklist dvb_usb_rtl28xxu' > /etc/modprobe.d/blacklist-rtl.conf
# modprobe -r dvb_usb_rtl28xxu   # (or replug)

## Layer 2 — Does SoapySDR see it?

This is the layer SDR Hunter actually talks to. Enumerate with the SoapySDR tool directly:

$ SoapySDRUtil --find

You should see a probe result with a driver= key (e.g. driver=bladerf, driver=rtlsdr). Full capability probe (sample-rate ranges, gains, bandwidths):

$ SoapySDRUtil --probe="driver=<yourdriver>"

If lsusb shows the device but SoapySDRUtil --find does not: the matching SoapySDR module isn’t installed or isn’t on the module path. Re-run ./install_drivers.sh, then check SoapySDRUtil --info lists your driver under “Available factories”.

Permissions check: if --find only works under sudo, the udev rule didn’t apply. Fix the rules (Layer 0 step 3) and replug — do not routinely run the app as root.

## Layer 3 — Does SDR Hunter see it?

Now use the app’s own enumeration, which goes through its DeviceManager:

$ python3 main.py --list-devices

Expected: SoapySDR available: True and your device listed with driver/label/ serial. If it prints the synthetic/mock device instead (“No hardware/SoapySDR detected; using synthetic device”), SoapySDR isn’t importable from this Python — you’re likely in a virtualenv that can’t see the system SoapySDR bindings. Either use the system Python, or create the venv with --system-site-packages.

For a deeper, layer-by-layer automated check, run the bundled diagnostic:

$ python3 tools/bringup_check.py
$ python3 tools/bringup_check.py --probe   # also does a short capture per device

## Layer 4 — First real capture (smoke test)

Launch the desktop GUI against real hardware (drop the mock env vars — they force the synthetic device):

$ ./launch.sh
# equivalently: python3 main.py --gui

In the Device panel:

- Select your real device (not “Mock”).

- Set a center frequency on a known-active band — FM broadcast 88–108 MHz is the easiest sanity check.

- Set Sample rate and Bandwidth within the device’s range (start conservative: 2.0–2.4 MS/s, 2.0 MHz bandwidth). The Bandwidth control clamps to your device’s profile automatically.

- Set gain to ~30–40 dB (or enable AGC) and start the stream.

Success looks like: a live spectrum with a noise floor and, on the FM band, clear station peaks; the waterfall scrolls. Tune to a strong FM station and the FM decoder should produce audio.

If the spectrum is flat/dead: wrong antenna for the band, gain too low, or the device is streaming but not tuned where you think. If it’s a wall of noise: gain too high (clipping) or you’re near strong interference — reduce gain.

## Layer 5 — Dual-RX, web dashboard & tuning

- Web dashboard: python3 main.py --web (or --both for GUI + web), then open http://localhost:8000. Confirm the WebSocket status indicators go green (/ws/spectrum, /ws/events, /ws/waterfall).

- Remote tune / RX routing: in the web tune form pick RX1 (scanner) vs RX2 (focus) and a frequency, then Focus. RX1 parks the scanner on that frequency (halting the sweep); release resumes the sweep. RX2 tunes the focus receiver independently. Verify each RX behaves as expected on real hardware. (Bandwidth is selectable in the desktop Device panel; the web tune form does not yet expose a bandwidth field.)

- Two-device setup: for genuine simultaneous dual-RX, attach two radios (or one 2-channel device like LimeSDR/USRP). Confirm both appear in --list-devices with distinct serials.

## Layer 6 — Decoders & ATAK (as needed)

- NOAA APT weather-sat: needs a real pass on 137 MHz with an appropriate antenna (V-dipole/QFH). Decoder runs at 20800 Hz, sync-aligned, false-color composite.

- Drone ID: tune the drone-ID bands and confirm detections populate the drone map.

- ATAK bridge: open the ATAK config dialog, set multicast/unicast + callsign

  - stale time, and use Test ping to confirm your ATAK/WinTAK endpoint receives CoT before relying on live detections.

## Quick troubleshooting matrix

| Symptom | Most likely layer | First thing to check |
| --- | --- | --- |
| Device missing from lsusb | 1 (USB) | Cable (data, not charge-only), powered USB 3 port |
| In lsusb, not in SoapySDRUtil --find | 2 (Soapy module) | Re-run install_drivers.sh; check factories |
| Works only with sudo | 0/2 (udev) | Reload udev rules + replug |
| App shows mock despite hardware present | 3 (Python env) | venv can’t see system SoapySDR bindings |
| Flat/dead spectrum | 4 (RF) | Antenna for band, gain, actual tuned freq |
| Wall of noise | 4 (RF) | Gain too high / nearby interference |
| Web WS indicators red | 5 (server) | Server running? Port 8000 free? firewall? |

## What can be validated here vs. only on your machine

This environment is headless with a mock device, so everything below the RF layer (imports, enumeration fallback, API routing, decoder unit paths) is already validated. Layers 1, 2, and 4–6 require the physical radio and RF signals and can only be confirmed on your hardware. Report back what --list-devices and tools/bringup_check.py print if anything doesn’t line up with the expected output above.